CCE-P1706 (Revelle RR1710) CTD Data

R. Goericke, SIO,

Sept 2017

This document describes the processing of CTD data acquired on cruise CCE-P1706.

Notes on Using CTD Data from CCE-P1706

The data are of two types, calibrated data and uncalibrated data.  The calibrated data are those for which it is assumed that reliable and stable calibrations exist.  These are pressure, depth, temperature, conductivity, salinity, light transmission, and ISUS nitrate.  The uncalibrated data are fluorescence, oxygen concentrations, and irradiance.  Uncalibrated data were made with sensors that had undergone calibrations in the past but the stability of the sensors’ response over the cruises or the relationship of sensor raw signal to desired variable could not be ascertained independently.  Even though every effort was made to assure that ‘uncalibrated data’ were correctly acquired and processed (see below), these data should not be used to make subtle inferences about differences between cycles and in particular cruises. The main value of these data is, and their original intended purpose was, that these be used to reveal qualitatively water column structure at each individual station. 
To summarize, the accuracy of uncalibrated data is unknown and it must be assumed that the precision and accuracy of these data differed between cycles and cruises!

CTD Setup

The Seabird con-file with instrument information and calibrations is given in appendix 2.  Corresponding sensor calibration information is in the directory …\DataAquFiles. 
Initial Sensor - Voltage channel numbers are given below.

Voltage Channel - Instrument

V0 – PIC 011

V1 – CST 1450

V2 – Turbidity Gain100

V3 – UVP   (UserP 0)

V4 – PAR

V5 – ISUS  (UserP 1)

V6 – oxygen, SBE 43

V7 – Flurometer, Seapoint

For some casts different Voltage channel configurations were used.  These are documented in the database table CTD_CastInfo.

UVP5  (Serial 201; Cal Date unkn):  Cal: A0: 13.224; A1: 912.

General Notes on Data Processing:  Data were processed using the program ‘Seabird Data Processing Version 7.21d’.  All raw data files, i.e. those produced by the SeaSave program, are stored in the directory …\ RawData. The .cnv and .btl files were further processed using a set of Matlab scripts that are in folder ‘*:\Documents\MATLAB\Cruises\CTD Scripts’.  These scripts are internally documented.

Raw Seabird Data Files located in  ‘…CTD\Raw Data’:

P1706C001.XMLCON	instrument configuration file used for the particular cast; the con-file report for one particular *.con file is given in appendix 2

P1706C001.hex	raw data file in hex format

P1706C001.bl	bottle information

Seabird Data Processing

A script was set up for batch processing of the CTD data (App. 2).  The Seabird modules settings are recorded as .psa files.  These files are in the folder ‘*:\Data\CTD\PSAFiles’. Settings are outlined below (settings slightly modified from CalCOFI settings and text below slightly modified from CalCOFI procedure outline, provided by J. Wilkinson).

a.	Datcnv: Output variables are – Scan Count; Julian Days; Pressure, Digiquartz [db]; Temperature [ITS-90,degC]; Conductivity[S/m]; Temperature 2,[ITS-90,degC]; Conductivity 2[S/m]; voltages 0 – 7; Oxygen Voltage, SBE43; SPAR/Surface Irradiance; PAR/Irradiance, Biospherical/Licor; Beam Attenuation, Chelsea/Seatech/Wetlabs CStar[1m]; Beam Transmission, Chelsea/Seatech/Wetlabs CStar[%]; Fluorescence, Seapoint; Latitude[deg]; Longitude[deg], Scan count.  Output Down & Upcasts; binary format; create cnv only  Used 33 as base latitude under miscellaneous if GPS lat is unavailable. Scan range offset -4, scan range 4. 
For Frontal crossings on-the-fly bottle triggering -0.5, 1 was used.

b.	Filter: Filter all files using Filter B = 0.15 on Pressure only (standard for 911+ Seabird).

c.	AlignCTD: align all files using 0.073 for secondary conductivity (only since the deck unit auto-offsets primary conductivity) and 4-6secs for oxygen (usually 4secs).

d.	Cell Thermal Mass: default settings of 0.03 & 7 for both conductivity sensors are fine

e.	WildEdit: apply to all fields using the default settings

f.	LoopEdit: Fixed minimum velocity; min CTD velocity 0.25 m/sec; Surf Soak Depth 20m, Min Soak 5 m, Max Soak 25 m.

g.	Derive: Depth[salt water,m]; Potential Temperature[ITS-90,degC]; Potential Temperature 2[ITS-90,degC]; Salinity[PSU]; Salinity 2[PSU]; Density[sigma-theta,KG/m^3]; Density 2[sigma-theta,KG/m^3]; Oxygen[SBE43, ml/l]; Oxygen[SBE43, umol/kg]; Oxygen[%saturation]; Dynamic Meters[10 J/Kg]; Specific Volume Anomaly[10^-8*m^3/Kg]; Temperature Difference 2-1[ITS-90,degC]; Salinity Difference[PSU]

h.	Split the cnv into down- and upcast.

i.	BinAvg the downcast files

j.	Asciiout the final data, include all measurements but beware Asciiout will crash if you include too many columns.  Exclude salinity & temperature differences, scan count, flag columns.

The following files were generated during this process and can be found in the directories, …\RawData\...,  …\ProcData\Btl and …\ProcData\Ascii\...’:

P1706C001.cnv 	hex data file generated and subsequently overwritten by modules Datcnv through Derive

P1706C001.ros	ascii file with data corresponding to those scans that were subsequently averaged to generate the .btl file

d(u)P1706C001.cnv	downcast (or upcast) data only derived from cce-p0810-001.cnv

d(u)P1706C001-bin.cnv	1m bin-averaged downcast hex data derived from dcce-p0810-001.cnv using Binavg

d(u)P1706C001-bin.asc	ascii file of 1 m bin-averaged downcast data; no header information

d(u)P1706C001-bin.hdr	header information for file dcce-p0810-001-bin.asc

d(u)P1706C001-nobin.asc	ascii file of 24 Hz downcast data; no header information

d(u)P1706C001-nobin.hdr	header information for file dcce-p0810-001-NoBin.asc

P1706C001.btl	ascii file with averaged bottle data, derived from .bl files and .ros files. File has header information.

CTD Data Generated:

Below a list of CTD data, primary and derived, that are in the final .cnv files (information extracted from file header information; see Seabird documentation for more detailed information on the data definitions). Note that not all these data categories are in all processed files, i.e. the ‘.asc’ files.

No.	Name	Description & Units

0	scan	Scan Count

1	timeS	Time, Elapsed [seconds]

2	prDM	Pressure, Digiquartz [db]

3	t090C	Temperature [ITS-90, deg C]

4	t190C	Temperature, 2 [ITS-90, deg C]

5	c0S/m	Conductivity [S/m]

6	c1S/m	Conductivity, 2 [S/m]

7	v0	Voltage 0

8	v1	Voltage 1

9	v2	Voltage 2

10	v3	Voltage 3

11	v4	Voltage 4

12	v5	Voltage 5

13	v6	Voltage 6

14	v7	Voltage 7

15	sbeox0V	Oxygen Voltage, SBE 43

16	spar	SPAR/Surface Irradiance [uEi /sqm sec]

17	par	PAR/Irradiance, Biospherical/Licor [uEi /sqm sec]

18	bat	Beam Attenuation, Wetlab CStar [1/m]

19	xmiss	Beam Transmission, Chelsea/Seatech/Wetlab CStar [%]

20	flSP	Fluorescence, Seapoint [V]

21	upoly1	Upoly 1, UVP [number of particles 100 to 1000 um ESD]

22	upoly0	Upoly 0, ISUS Nitrate Sensor [uM NO3]

23	latitude	Latitude [deg]

24	longitude	Longitude [deg]

25	scan	Scan Count

26	depSM	Depth [salt water, m]

27	potemp090C	Potential Temperature [ITS-90, deg C]

28	potemp190C	Potential Temperature, 2 [ITS-90, deg C]

29	sal00	Salinity [PSU]

30	sal11	Salinity, 2 [PSU]

31	sigma-é00	Density [sigma-theta, Kg/m^3]

32	sigma-é11	Density, 2 [sigma-theta, Kg/m^3]

33	sbeox0Mm/Kg	Oxygen, SBE 43 [umol/Kg], WS	2

34	sbeox0PS	Oxygen, SBE 43 [% saturation], WS	2

37	T2-T190C	Temperature Difference, 2 - 1 [ITS-90, deg C]

38	secS-priS	Salinity Difference, 2 - 1 [PSU]

39	flag	flag

Generation of Database Tables:

Files d*-bin.hdr, d*-bin.asc and *.btl provide the data for database tables CTD_CastInfo, CTD_BottleData and CTD_Downcast, respectively.  Data are extracted from these files using Matlab scripts (see below).

Cast_Info – information extracted from the headers of .asc files using CTD_info.m.  The resulting file was e.g., CastInfo_Cy1.txt.  These files were imported into the Access table CTD_CastInfo.

CTD_BottleData – data extracted from .btl files using CTD_ bottle.m.  The resulting file was e.g., BottleData_Cy1.txt.  These files were imported into the Access table CTD_BottleData.

CTD_Downcast- data extracted from .asc files using CTD_ data.m.  The resulting file was e.g., Downcastdata_Cy1.txt.  These files were imported into the Access table CTD_Downcast.

Post-Processing QC and Calibrations:

General QC:

Differences between CTD channel 1 and 2 Temp and Sal were used to assess the proper functioning of the two pump channel.  Any problems encountered are documented in the database table CTD_CastInfo along with a short description of the action taken to address the problems.

Comparision – CTD Sal. vs Bottle Sal.

39 salinity samples were collected at sea for Autosal analysis (P1706 salt data.xlsx). The comparison was carried out for samples located away from strong Sal gradients; i.e. samples collected at the surface and depths between 200 and 1000 m.  This is based on avg(CTD_Sal00, CTD_Sal11) minus Bottle_Sal.  The plot of these values is shown to the right.

The average value of the difference was -0.006 +/- 0.006.  The corresponding value of CTD_Sal00 minus CTD_Sal11 is -0.0005 +/- 0.0009.  The difference between CTD and Bottle salinities is not large enough to warrant a correction.  There is no trend with time for these differences.

ISUS Nitrate Calibration

ISUS V2 was not recorded. All bottle nutrient data were used for the ISUS calibration. A cubic polynomial was found to have the best fit to the data:

ISUS_NO3 =  -2.869 * ISUS_V.^3 + 12.77 * ISUS_V.^2 + 9.884 * ISUS_V -4.425;

fitresult(x) = p1*x^3 + p2*x^2 + p3*x + p4

Coefficients (with 95% confidence bounds):

p1 =      -2.869  (-4.358, -1.38)

p2 =       12.77  (7.913, 17.62)

p3 =       9.884  (5.108, 14.66)

p4 =      -4.425  (-5.813, -3.037)

PAR and CPAR:

All values of PAR less than 0.1 uEi and associated values of SPAR were deleted.

CTD Plots:

Single Cast

These plots are in the folder …\ProcData\Plots and were generated using Seabird’s Seaplot module.  Data plotted are downcast data only, both binned and un-binned data.  The source of the data is given in the plot title which gives the name of the data file used, e.g. dp1706c001-bin.cnv and dp1706c001.cnv.

Contour Plots

These plots were generated from data extracted from the CTD database and stored in directory * \ContourPlots.  The data at times were spot-edited to remove outliers that had not been removed by Seabird modules.  The plots were generated using various MatLab scripts. CTD data for individual cycles are plotted against ‘cycle time’.  Cycle time 0.0 is the beginning of the GMT day on which the cycle was begun.  Thus 2 am casts have cycle times of ~ 0.4, 1.4, 2.4 … and daylight hours have GMT times of ~ 14:00 to 24:00 or cycle times of 0.5 to 1.0, etc.

Appendix 1 – Cast Notes

| CTD_CastInfo |  |  |  |
| --- | --- | --- | --- |
| Cruise | Cast | Notes | Actions |
| CCE-P1706 | 1 | SPAR off | deleted SPAR values |
| CCE-P1706 | 2 | Santa Barbara Basin (Nuts are off at depth). no PAR sensor SPAR off | deleted SPAR values |
| CCE-P1706 | 3 | no PAR sensor |  |
| CCE-P1706 | 4 |  |  |
| CCE-P1706 | 5 |  |  |
| CCE-P1706 | 6 | Trans Volt was corrected by a factor of + 0.132 (dirty Trans windows) |  |
| CCE-P1706 | 7 |  |  |
| CCE-P1706 | 8 |  |  |
| CCE-P1706 | 9 |  |  |
| CCE-P1706 | 10 |  |  |
| CCE-P1706 | 11 |  |  |
| CCE-P1706 | 12 |  |  |
| CCE-P1706 | 13 |  |  |
| CCE-P1706 | 14 | large T1-T2 differences. C1-C2 are fine. |  |
| CCE-P1706 | 15 | large T1-T2 differences. C1-C2 are fine. |  |
| CCE-P1706 | 16 | large T1-T2 differences. C1-C2 are fine. |  |
| CCE-P1706 | 17 | large T1-T2 differences. C1-C2 are fine. |  |
| CCE-P1706 | 18 |  |  |
| CCE-P1706 | 19 |  |  |
| CCE-P1706 | 20 |  |  |
| CCE-P1706 | 21 |  |  |
| CCE-P1706 | 22 |  |  |
| CCE-P1706 | 23 |  |  |
| CCE-P1706 | 24 |  |  |
| CCE-P1706 | 25 |  |  |
| CCE-P1706 | 26 |  |  |
| CCE-P1706 | 27 |  |  |
| CCE-P1706 | 28 |  |  |
| CCE-P1706 | 29 |  |  |
| CCE-P1706 | 30 |  |  |
| CCE-P1706 | 31 |  |  |
| CCE-P1706 | 32 |  |  |
| CCE-P1706 | 33 |  |  |
| CCE-P1706 | 34 |  |  |
| CCE-P1706 | 35 | Deep cast, no PAR sensor |  |
| CCE-P1706 | 36 | TC_Channel 2 was clogged below 70 m. | Sal00_Q set to true |
| CCE-P1706 | 37 | large T1-T2 differences. C1-C2 are fine. |  |
| CCE-P1706 | 38 | large T1-T2 differences. C1-C2 are fine. | Sal00_Q set to true |
| CCE-P1706 | 39 | large T1-T2 differences. C1-C2 are fine. ISUS did not work | deleted all valuse for ISUS |
| CCE-P1706 | 40 |  |  |
| CCE-P1706 | 41 |  |  |
| CCE-P1706 | 42 | UVP questionable - values too high |  |
| CCE-P1706 | 43 | Bottle 19 was not fired. |  |
| CCE-P1706 | 44 |  |  |
| CCE-P1706 | 45 |  |  |
| CCE-P1706 | 46 |  |  |
| CCE-P1706 | 47 |  |  |
| CCE-P1706 | 48 |  |  |
| CCE-P1706 | 49 |  |  |
| CCE-P1706 | 50 |  |  |
| CCE-P1706 | 51 |  |  |
| CCE-P1706 | 52 |  |  |
| CCE-P1706 | 53 |  |  |
| CCE-P1706 | 54 |  |  |
| CCE-P1706 | 55 |  |  |
| CCE-P1706 | 56 |  |  |
| CCE-P1706 | 57 | Deep cast, no PAR sensor |  |
| CCE-P1706 | 58 | PAR sensor was not plugged in |  |
| CCE-P1706 | 59 |  |  |
| CCE-P1706 | 60 |  |  |
| CCE-P1706 | 61 |  |  |
| CCE-P1706 | 62 |  |  |
| CCE-P1706 | 63 |  |  |
| CCE-P1706 | 64 |  |  |
| CCE-P1706 | 65 |  |  |
| CCE-P1706 | 66 |  |  |
| CCE-P1706 | 67 |  |  |
| CCE-P1706 | 68 |  |  |
| CCE-P1706 | 69 |  |  |
| CCE-P1706 | 70 | 12 place Carousel. 24 Bottles fired in pairs |  |
| CCE-P1706 | 71 | 12 place Carousel. 24 Bottles fired in pairs |  |
| CCE-P1706 | 72 | 12 place Carousel. 24 Bottles fired in pairs |  |
| CCE-P1706 | 73 | 12 place Carousel. 24 Bottles fired in pairs |  |
| CCE-P1706 | 74 | 12 place Carousel. 24 Bottles fired in pairs |  |
| CCE-P1706 | 75 | 12 place Carousel. 24 Bottles fired in pairs |  |
| CCE-P1706 | 76 | w/o PAR. 12 place Carousel. 24 Bottles fired in pairs |  |
| CCE-P1706 | 77 | w/o PAR. 12 place Carousel. 24 Bottles fired in pairs |  |
| CCE-P1706 | 78 | w/o PAR. 12 place Carousel. 24 Bottles fired in pairs |  |
| CCE-P1706 | 79 | w/o PAR. 12 place Carousel. 24 Bottles fired in pairs |  |
| CCE-P1706 | 80 | w/o PAR. 12 place Carousel. 24 Bottles fired in pairs |  |
| CCE-P1706 | 81 | w/o PAR. 12 place Carousel. 24 Bottles fired in pairs |  |
| CCE-P1706 | 82 | w/o PAR. 12 place Carousel. 24 Bottles fired in pairs |  |
| CCE-P1706 | 83 | w/o PAR. 12 place Carousel. 24 Bottles fired in pairs |  |
| CCE-P1706 | 84 | w/o PAR. 12 place Carousel. 24 Bottles fired in pairs |  |
| CCE-P1706 | 85 | Four Transm. 24 Bottles fired in pairs |  |
| CCE-P1706 | 86 | Four Transm. 24 Bottles fired in pairs |  |
| CCE-P1706 | 87 | Four Transm. 24 Bottles fired in pairs |  |
| CCE-P1706 | 88 | Four Transm. 24 Bottles fired in pairs.TC_Cha 1 bad below 250 m | Sal00_Q set to true below 259 m |
| CCE-P1706 | 89 | Four Transm. 24 Bottles fired in pairs |  |
| CCE-P1706 | 90 | Four Transm. 24 Bottles fired in pairs |  |

Appendix 2 – CTD Con File

Date: 06/02/2017

Instrument configuration file: C:\Data\CTD\P1706\RawData\P1706C001.XMLCON

Configuration report for SBE 911plus/917plus CTD

------------------------------------------------

Frequency channels suppressed : 0

Voltage words suppressed      : 0

Computer interface            : IEEE-488 (GPIB)

Deck unit                     : SBE11plus Firmware Version >= 5.0

Scans to average              : 1

NMEA position data added      : Yes

NMEA depth data added         : No

NMEA time added               : No

NMEA device connected to      : deck unit

Surface PAR voltage added     : Yes

Scan time added               : Yes

1) Frequency 0, Temperature

Serial number : 6141

Calibrated on : 26-Apr-2017

G             : 4.39067304e-003

H             : 6.41213054e-004

I             : 2.33864811e-005

J             : 2.24359842e-006

F0            : 1000.000

Slope         : 1.00000000

Offset        : 0.0000

2) Frequency 1, Conductivity

Serial number : 1919

Calibrated on : 07-Apr-17

G             : -3.96233040e+000

H             : 5.21736839e-001

I             : -9.88191833e-004

J             : 7.80444247e-005

CTcor         : 3.2500e-006

CPcor         : -9.57000000e-008

Slope         : 1.00000000

Offset        : 0.00000

3) Frequency 2, Pressure, Digiquartz with TC

Serial number : 0830

Calibrated on : 13-MAR-2017

C1            : -4.071245e+004

C2            : -7.704916e-001

C3            : 1.234291e-002

D1            : 3.170469e-002

D2            : 0.000000e+000

T1            : 3.008932e+001

T2            : -5.622765e-004

T3            : 4.462390e-006

T4            : -4.719337e-009

T5            : 0.000000e+000

Slope         : 1.00000000

Offset        : 0.00000

AD590M        : 1.290360e-002

AD590B        : -8.201883e+000

4) Frequency 3, Temperature, 2

Serial number : 6142

Calibrated on : 26-Apr-2017

G             : 4.36967414e-003

H             : 6.39807061e-004

I             : 2.32713552e-005

J             : 2.28941901e-006

F0            : 1000.000

Slope         : 1.00000000

Offset        : 0.0000

5) Frequency 4, Conductivity, 2

Serial number : 3578

Calibrated on : 07-Mar-17

G             : -9.48645513e+000

H             : 1.15092608e+000

I             : -1.41494124e-003

J             : 1.49145616e-004

CTcor         : 3.2500e-006

CPcor         : -9.57000000e-008

Slope         : 1.00000000

Offset        : 0.00000

6) A/D voltage 0, Transmissometer, WET Labs C-Star

Serial number : CSTAR-A

Calibrated on :

M             : 1.0000

B             : 0.0000

Path length   : 0.250

7) A/D voltage 1, Transmissometer, WET Labs C-Star, 2

Serial number : CSTAR B

Calibrated on :

M             : 1.0000

B             : 0.0000

Path length   : 0.250

8) A/D voltage 2, Turbidity Meter, Seapoint

Serial number :

Calibrated on :

Gain setting  : 100 x

Scale factor  : 1.000

9) A/D voltage 3, User Polynomial

Serial number : 201

Calibrated on : YYYYMMDD

Sensor name   : UVP5

A0            : 13.22400000

A1            : 912.00000000

A2            : 0.00000000

A3            : 0.00000000

10) A/D voltage 4, PAR/Irradiance, Biospherical/Licor

Serial number        : 70629

Calibrated on        : 05-May-2016

M                    : 1.00000000

B                    : 0.00000000

Calibration constant : 10799136069.11000100

Multiplier           : 1.00000000

Offset               : -0.09360000

11) A/D voltage 5, User Polynomial, 2

Serial number : ISUS

Calibrated on : 2107-04 CalCOFI

Sensor name   : ISUS

A0            : -6.00000000

A1            : 26.00000000

A2            : 0.00000000

A3            : 0.00000000

12) A/D voltage 6, Oxygen, SBE 43

Serial number : 0060

Calibrated on : 14-Sep-16

Equation      : Sea-Bird

Soc           : 4.50580e-001

Offset        : -5.00000e-001

A             : -3.70830e-003

B             : 2.37010e-004

C             : -3.38840e-006

E             : 3.60000e-002

Tau20         : 9.30000e-001

D1            : 1.92634e-004

D2            : -4.64803e-002

H1            : -3.30000e-002

H2            : 5.00000e+003

H3            : 1.45000e+003

13) A/D voltage 7, Fluorometer, Seapoint

Serial number : SCF3781

Calibrated on :

Gain setting  : 10 x, 0-15 µg/l

Offset        : 0.000

14) SPAR voltage, Unavailable

15) SPAR voltage, SPAR/Surface Irradiance

Serial number     : 6368

Calibrated on     : 16-Mar-2017

Conversion factor : 1726.30000000

Ratio multiplier  : 1.00000000

Scan length                   : 44

Appendix 3 – SBE Batch Files

CTDBatchP1706

@	Command file for processing Seabird CTD_Data

@	Open up cmd.exe by typing cmd into the 'Search programs ..." window under Start and call the script by pasting the line below into the command window

@	sbebatch C:\Data\CTD\P1706\CTDBatchP1706.txt

@	The line above is the command line to execute SBE CTD_Data batch processing.

@	The line consists of:

@	1. the program to be used, i.e. 'sbebatch'

@	2. the directory location of the batch file, e.g. 'D:\CCEP0810\CTD\RawData\CTDBatch.txt'

@	3. a string of variables to be used in the processing of the data. These are referred

@	    to in the CTDBatch.txt file below in the order these appear as %1, %2, ...

@	Datcnv /pC:\Data\CTD\P1706\PSAFiles\Datcnv.psa  /iC:\Data\CTD\P1706\RawData\*.hex  /oC:\Data\CTD\P1706\ProcData\ProcCnv

Filter /pC:\Data\CTD\P1706\PSAFiles\Filter.psa  /iC:\Data\CTD\P1706\RawData\*.cnv  /oC:\Data\CTD\P1706\ProcData\ProcCnv

Alignctd /pC:\Data\CTD\P1706\PSAFiles\AlignCTD.psa  /iC:\Data\CTD\P1706\ProcData\ProcCnv\*.cnv /oC:\Data\CTD\P1706\ProcData\ProcCnv

Celltm /pC:\Data\CTD\P1706\PSAFiles\CellTM.psa  /iC:\Data\CTD\P1706\ProcData\ProcCnv\*.cnv /oC:\Data\CTD\P1706\ProcData\ProcCnv

Wildedit /pC:\Data\CTD\P1706\PSAFiles\WildEdit.psa /iC:\Data\CTD\P1706\ProcData\ProcCnv\*.cnv /oC:\Data\CTD\P1706\ProcData\ProcCnv

Loopedit /pC:\Data\CTD\P1706\PSAFiles\LoopEdit.psa  /iC:\Data\CTD\P1706\ProcData\ProcCnv\*.cnv /oC:\Data\CTD\P1706\ProcData\ProcCnv

Derive /pC:\Data\CTD\P1706\PSAFiles\Derive.psa /iC:\Data\CTD\P1706\ProcData\ProcCnv\*.cnv  /oC:\Data\CTD\P1706\ProcData\ProcCnv

Split /pC:\Data\CTD\P1706\PSAFiles\Split.psa  /iC:\Data\CTD\P1706\ProcData\ProcCnv\*.cnv /oC:\Data\CTD\P1706\ProcData\ProcCnv

Asciiout /pC:\Data\CTD\P1706\PSAFiles\ASCII_Out.psa /iC:\Data\CTD\P1706\ProcData\ProcCnv\dp1706c???.cnv /oC:\Data\CTD\P1706\ProcData\Ascii /a-nobin

Binavg /pC:\Data\CTD\P1706\PSAFiles\BinAvg.psa  /iC:\Data\CTD\P1706\ProcData\ProcCnv\dp1706c???.cnv /oC:\Data\CTD\P1706\ProcData\ProcCnv  /a-bin

Asciiout /pC:\Data\CTD\P1706\PSAFiles\ASCII_Out.psa /iC:\Data\CTD\P1706\ProcData\ProcCnv\dp1706c???-bin.cnv /oC:\Data\CTD\P1706\ProcData\Ascii

Bottlesum /pC:\Data\CTD\P1706\PSAFiles\BottleSum.psa  /iC:\Data\CTD\P1706\RawData\*.ros  /oC:\Data\CTD\P1706\ProcData\Btl

CTDBatchP1706Plots

@	Command file for processing Seabird CTD_Data

@	Open up cmd.exe by typing cmd into the 'Search programs ..." window under Start and call the script by pasting the line below into the command window

@	sbebatch C:\Data\CTD\P1706\CTDBatchP1706Plots.txt

@

@	The line above is the command line to execute SBE CTD_Data batch processing.

@	The line consists of:

@	1. the program to be used, i.e. 'sbebatch'

@	2. the directory location of the batch file, e.g. 'D:\CCEP0810\CTD\RawData\CTDBatch.txt'

@	3. a string of variables to be used in the processing of the data. These are referred

@	   to in the CTDBatch.txt file below in the order these appear as %1, %2, ...

Seaplot /pC:\Data\CTD\PSAFiles\SeaPlot-Hydro.psa /iC:\Data\CTD\P1706\ProcData\ProcCnv\d?????????????.cnv /oC:\Data\CTD\P1706\ProcData\Plots /a-1_Hydro

Seaplot /pC:\Data\CTD\PSAFiles\SeaPlot-Particles.psa /iC:\Data\CTD\P1706\ProcData\ProcCnv\d?????????????.cnv /oC:\Data\CTD\P1706\ProcData\Plots /a-3_Particles

Seaplot /pC:\Data\CTD\PSAFiles\SeaPlot-Light.psa /iC:\Data\CTD\P1706\ProcData\ProcCnv\d?????????????.cnv /oC:\Data\CTD\P1706\ProcData\Plots /a-2_Light

Seaplot /pC:\Data\CTD\PSAFiles\SeaPlot-Trans4_a.psa /iC:\Data\CTD\P1706\ProcData\ProcCnv\d?????????????.cnv /oC:\Data\CTD\P1706\ProcData\Plots /a-4_Trans_a

Seaplot /pC:\Data\CTD\PSAFiles\SeaPlot-Trans4_b.psa /iC:\Data\CTD\P1706\ProcData\ProcCnv\d?????????????.cnv /oC:\Data\CTD\P1706\ProcData\Plots /a-4_Trans_b